Providing Location-privacy in Opportunity Mobile Social Network

by

Rui Huang

A Thesis submitted to the Faculty of Graduate and Postdoctoral Studies

in partial fulfillment of the requirements for the degree of

MASTER OF APPLIED SCIENCE

in Electrical and Computer Engineering

Ottawa-Carleton Institute of Electrical and Computer Engineering

University of Ottawa

Ottawa, Canada

September 3, 2017

© Rui Huang, Ottawa, Canada, 2017

Abstract

Todo

Acknowledgements

Todo

Table of Contents

Abstract	i

Acknowledgements	ii

Table of Contents	iii

List of Figures	vi

List of Tables	vii

Chapter 1 Introduction	1

Chapter 2 Background and Related Work	3

Chapter 3 Multi-Hop Location-Privacy Protection	4

3.1. System Model	4

3.2. Details of MHLPP	5

3.3. Requirement parameters	9

3.4. Privacy Analysis	10

3.5. Complexity discussion	11

3.6. Performance Analysis	12

3.7. Query success ratio	14

Chapter 4 Appointment Card Protocol	21

4.1. System Model	21

4.2. Appointment Card Protocol Overview	21

4.3. Appointment Card	23

4.4. AC life cycle	25

4.5. System parameters	26

4.5.1. Obfuscation distance	26

4.5.2. Friends obfuscation distance	27

4.5.3. Distributing segment	28

4.5.4. Generating Period	29

4.5.5. AC timeout	29

4.6. Protocol Details	29

4.6.1. Generating appointment cards	29

4.6.2. Exchange distributing appointment cards	30

4.6.3. Exchange ready appointment cards	31

4.6.4. Sending queries	32

4.6.5. Sending replies	33

4.6.5.1. The LBSP part	33

4.6.6. The first agent	34

4.6.7. Intermediate agents	35

4.6.8. The last agent	36

4.7. Appointment number	38

4.7.1. Creator appointment number	38

4.7.2. Agent appointment number	38

4.7.3. Timeout	39

4.8. Experiment	39

4.8.1. Average query success ratio	40

4.8.2. Average reply success ratio	43

4.8.3. Total number of query relay	44

4.8.4. Memory cost	45

4.8.5. Distributing appointment cards	47

4.9. Exchange sending process	50

4.9.1. Generate new ACs	51

4.9.2. Pick dispensed ACs	51

4.9.3. Pick ready ACs	52

4.9.4. Send ACs	52

4.9.4.1. Get the number of ACs	53

4.9.4.2. Check duplicated appointment numbers	53

4.10. Exchange receiving process	53

4.11. Transmit replies	54

4.12. Example	56

Chapter 5 Vertex reduce algorithm	60

5.1. Ignorable vertex and reserved vertex	60

5.2. Vertex reducing	62

5.2.1. Remove ignorable vertices	62

5.2.2. Tidy reserved vertices connections	62

5.2.3. Iterations	63

5.3. Vertex assembling	64

Chapter 6 Finding nodes in a range	66

6.1. Static nodes	66

6.2. Mobile nodes	67

6.3. Complexities	68

REFERENCES:	69

List of Figures

Figure 4.1 AC exchange process (send)	7

Figure 4.2 AC exchange process (receive)	10

Figure 4.3 Transmit reply	11

List of Tables

Table 4.1 Symbols	23

Table 4.2 Appointment Card	25

Table 4.3 Important system parameters	26

Table 4.4 relay table entries	31

Table 4.5 reply table entries of the first agent	34

Table 4.6 reply table entries of the second agent	35

Table 4.7 reply table entries of the last agent	37

Table 4.9 AC states	50

Table 4.10 AC storages	50

Table 4.11 Agent_list entries	53

Table 4.12 Alice and Bob exchange ACs	56

Table 4.13 Bob and Charlie exchange ACs	56

Table 4.14 Charlie and David exchange ACs	57

Table 4.15 The exchange history of ACx	57

Table 4.16 Users' agent_list	58

Table 4.17 Important information in the reply messages	58

Introduction

Location privacy is becoming a major concern in OMSNs which can be viewed as Delay Tolerant Networks (DTNs) [1], exhibiting lack of continuous connectivity. More specifically, in OMSNs, it is not necessary for senders to have an end-to-end routing path to their destinations. Nodes make contacts when they encounter each other. Location-Based Services (LBSs) are common applications in OMSNs and are widely used in military,  government organizations, emergency services and many commercial sectors [2], especially after the proliferation of localization technologies, like GPS. LBS users often send their location to LBS providers/servers. Many people access LBSs with their portable devices, as a result, their locations are also bound to their devices. In this case, LBS users face a continuous risk that their location may be leaked from the LBS applications. This makes people unwilling to use LBSs. Thus, location privacy protection has become a critical issue in LBS applications.

Early location privacy protection methods, such as obfuscation algorithms, generate anonymized areas for the original LBS requesters so that the requesters are mixed with a group of other LBS users [3]. Users send an anonymized area instead of their exact coordinate to the LBS providers when launching a LBS request. After that, social ties are incorporated into obfuscation algorithms to improve or protect location privacy. For example, the authors in [4] use social ties to determine trustable friends who could be chosen as intermediaries to forward obfuscation queries. The authors in [5] and [6] present algorithms which aim at improving delivery performance. However, compared to [4], the query success ratios of [5] and [6] have been shown to increase only by 5% and 11% respectively, which are not significant.

These papers assume that attackers can access the LBS servers, which enables them to learn LBS users’ identities and locations. If a user sends a query to a LBS server with his real identity and location (e.g., a query asking for a path from his current location to a certain place), attackers can locate the user quite easily. Therefore, hiding the original requester’s real identity and the location is the focus of our work.

Inspired by [5] and [7] which use the social network for messages forwarding in mobile ad hoc networks, we propose a distributed location-privacy algorithm, called MHLPP, to guarantee location-privacy and achieve a higher query success ratio. The introduction of social networks enables us to hide the original requester’s information behind his friends. When a user wants to send a query, he starts to look for friends based on information in his social network. He sends his query to the first encountered friend who is then responsible for forwarding the query to the intended location. This friend can also pass the query to one of his friends when they encounter. When the distance between the user carrying this query and the original requester exceeds a specified threshold, the user sends the query to the LBS server directly without having to find a friend to pass on. At that time, he also replaces the original requester’s information with its own identity and location, which enables the LBS server to receive the query without any information about the original requester. After receiving the query, the LBS server replies to the last friend (the user sending the query to the LBS) who then transmits it to the original requester.

MHLPP contains an obfuscation phase and a free phase. The process of finding friends for forwarding before the last friend sends the query out is called the obfuscation phase. In the free phase, the friend holding the query simply sends it to the LBS server, replacing the original requester’s identity with its own. The authors in [5] take a similar approach but differ from our approach as follows. Instead of finding k friends to finish the obfuscation phase as in case of [5], MHLPP takes the query to a place a specified distance away from the original requester. Moreover, [5] only selects friends among its neighbors while MHLPP selects friends among one-hop and multi-hop neighbors. This improvement enables MHLPP to gain a higher query success ratio when there are fewer friends in the network. In order to provide a secure communication among requesters and friends, especially multi-hop neighbor friends, encryption algorithms are used in MHLPP, unlike [5]. Our simulation results show that both one-hop and multi-hop connection between friends are acceptable in MHLPP while preventing un-trusted intermediate users from knowing the content of the queries.

The rest of this paper is organized as follows. Section II presents some related protocols in privacy-protection. The process of MHLPP is described in Section III. Experimental results are provided Section IV followed by some concluding remarks.

Background and Related Work

Users face risks of information breach when they access a semi-trusted LBS provider, because anyone who has access to data in LBSs is able to steal and misuse LBS users’ location-privacy. Considering that  LBSs rely on location-aware computing, it is unavoidable to leak users' location from LBSs. Therefore, balancing “these two competing aims of location privacy and location awareness” [8] is always a challenge.

Some early solutions, like [9] and [3], generate a specific area based on k-anonymity [10] for each user who needs to send queries. For example, [9] gives a rectangle as an anonymized area, in which all nodes form a group to hide the original requester. But, it requires at least k connected agents to complete its obfuscation process. [3] uses a central anonymity server as a mix router. As a result, it is necessary for each node to have a continuous connection to the server. That is hard to achieve in a sparse DTN. With a similar problem as in [3], [11] employs a matchmaker which is used to match users and advertisements, then users can achieve anonymization of their identities and locations from the matchmaker. However, the matchmaker is a high-risk in the network, because it collects so much private information. In the work in [12], exact locations and requests from clients are replaced by a location anonymization engine before they arrive at LBS providers. Since the anonymization engine learns all exact locations and requests, it becomes a better target for malicious attack.

There are protocols with more servers. The servers are settled in the network and each one of them takes charge of a certain area. [13] uses roadside units (RSUs) as mix servers in a vehicular DTN, and the destination is encrypted during forwarding, so eavesdropping queries cannot help attackers to locate users. But deploying the RSUs is not always feasible. In [14], sensor nodes which are scattered throughout the network provide anonymized locations for users. Since the sensor nodes’ coverage should possess a non-overlapping characteristic, it is difficult to deploy them in real-world. Besides, the mix servers and sensor nodes might be more prominent targets than LBS providers.

However, the system could also be a distributed one, like protocols [4], [5] and [6]. The obfuscation processes are performed by each separated node independently without any help from a third-party device. The utilization of label as in [15] makes it easier for nodes to mix themselves into a group, which is a significant difference from these protocols and the previous ones. Algorithms above use groups instead of an area to protect users. [5] imports 2 concepts: the obfuscation phase and the free phase. In the obfuscation phase, queries must be transmitted between friends for k times. When there are only a few friends in the network, it is hard for a node to find an available next hop in obfuscation phase. [5] and [6] attempt to improve [4]’s performance in the obfuscation phase, which might be a safety tradeoff, because some ineligible users in [4] are chosen as friends based on the additional standards imported by [5] and [6].

Multi-Hop Location-Privacy Protection

System Model

Our network architecture consists of two main entities: Users and LBS Providers (LBSPs). Due to the introduction of the social network, the users’ social information can be used in obfuscation forwarding process. Based on available information in the social network, the relationship between two users can be considered as friends or strangers. The user who makes a query to an LBSP will be called the original requester while the others are called intermediate users. LBSPs are located in fixed locations and their coordinates are known by all users when users join the network. Attackers are assumed to be able to access LBSPs, and attempt to locate original requesters. We assume that the LBSPs are semi-trusted and the strangers are un-trusted. We also assume that both entities have sufficient resources, like computational capability, storage and battery power.

Since two friends could be a pair of multi-hop neighbors, users can leverage Optimized Link State Routing Protocol [17] to seek friends continuously after entering the network, so that they can recognize each other and make contact in time. When a user carries an obfuscation phase query, he might send the query to a multi-hop friend through several strangers. In this case, a secure communication is necessary for between them, so that they must send the query encrypted to prevent strangers from learning anything about the query. Each user obtains a pair of asymmetric keys (public and secret key) before he joins the network from a certificate authority using well-regarded techniques, like in [16]. Whenever a user detects a new friend, he sends a request to the friend asking for his public key. In this way, a user can get his friends’ public key when they encounter each other. Even though several strangers can be active in the obfuscation phase, the queries can still be securely sent to the user’s friend.

The relationship strength is often “a hidden effect of nodal profile similarities” [18]. Let SVi,j denote a value of relationship strength which user i determines whether user j is an acceptable friend based on the relationship strength. For every pair of users (i and j), we assume that there is an SVi,j SVi,j. If SVi,j SVi,j is bigger than a specific friend threshold TminTmin, set by the original requester, user j is considered as a friend of user ii; otherwise, it will be treated as a stranger. The notations used in this paper and their meanings are shown in Table I.

Details of MHLPP

| MHLPP aims to protect the original requester’s (N0’s) location-privacy using an obfuscation path. In other words, a query q which needs to be obfuscated must go through a series of friends after it leaves N0. The whole process includes two parts: the obfuscation phase and the free phase. In the former phase, q is only transmitted among friends, until it is sent to Parameter | Meanings |
| --- | --- |
| N0 | the original requester |
| Ni | if i > 0, it denotes the friend chosen by Ni-1. If i = 0, it is N0. |
| Nf | the last friend who handles the obfuscation query |
| Nd | the destination or the LBSP |
| Ki | the public key of Ni |
| Si | the secret key of Ni Ni |
| q | a query of N0 |
| rq | the requirement for the query q |
| msg | a message contains q and rq |
| Emsgi | the encrypted msg msgusing Ki |
| Sid | the original requester’s identity |
| Did | the destination’s identity |
| Ls | the location of N0 when it sends the query to N1 |
| Rp | the inner radius of the obfuscation area |
| Rs | the external radius of the obfuscation area |
| Tmin | the social value bound for friends |
| Cmax | the extra path limit in each obfuscation forward |

an area called “obfuscation area”. At the end of that phase the last friend Nf replaces all N0’s information by its own and forwards q with an arbitrary DTN forwarding protocol, like the one suggested in [19]. In this case, what attackers can learn from the database in LBSP is Nf’s information, so they can hardly infer the original requester’s identity and location based on that information. The free phase starts when the query q is forwarded by Nf  and ends when it reaches the LBSP.

Because Nf is the only identity the LBSP knows, the LBSP has no choice other than replying to the last friend Nf when it receives the obfuscated query. The reply can be delivered with an arbitrary DTN routing protocol as the free phase query does. The friend Nf should remember who is the real destination (N0) of this reply, then he transmits it to N0. In this way, N0 is able to send a query q to an LBSP while not exposing his own information.

The obfuscation phase of a query q starts when the query leaves the original requester N0. When a user is holding an obfuscation phase query, he starts sensing connected friends continuously, which enables it to communicate with one-hop or multi-hop neighbor friends. Even though users in the mobile network use OLSR protocol [17] to detect others automatically, they do not communicate with their friend unless they have a requirement to send an obfuscation query. Also, they do not ask their friends for public keys. Therefore, carrying an obfuscation phase query requires a user to execute MHLPP algorithm.

When N0 finds the first available friend N1, he asks N1 for a public key K1, which will enable him to encrypt his query using K1. That prevent others, e.g. strangers, from learning information in the message msg that that contains both q and rq. rq is N0’s requirement for q, which is always sent with the query q and remains constant until the end of the obfuscation phase (we discuss this in part C). Friends who get the query can infer q’s obfuscation area based on parameters Rp, Rs and Ls in rq, which is a ring with inner radius Rp, external radius Rs and center Ls. Before N0 sends the query q to his friend, he initializes parameter Ls to his current location and encrypts msg using K1 to get Emsg1 which is what N0 sends to N1.

The destination of Emsg1 is N1, which is a plaintext in Emsg1, so that other intermediate users (strangers) can help N0 forward Emsg1 to N1. In this step, strangers transmit Emsg1 using the OLSR protocol if N1 is a multi-hop neighbor of N0. Strangers learn nothing other than the identity of N1, because both q and rq are encrypted. They cannot help attackers locate N0 because they do not know Sid (the identity of N0) and Did (the LBSP) included in rq.

When N1 receives Emsg1, he decrypts it with its secret key S1 to get q and rq. If q is already in the obfuscation area defined in rq (i.e., it is already in the ring), the query q finishes its the obfuscation phase. Then N1 replaces all information of N0 with his own. For example, the Sid is replaced with N1. If a location is necessary for the LBS, N1 uses his own current location and records this change in his memory before initiating the free phase. A free phase query can be then be forwarded to the destination (i.e. LBSP).

If q is still in the obfuscation phase (not in the ring), N1 performs similar actions just as N0 expect modifying rq. Another difference is that instead of finding friend randomly, N1 would seek for a friend who is nearer in the obfuscation area, and so will the following friend.

The detailed algorithm is explained in Algorithm 1 where Nx is a neighbor of Ni who is carrying the query q. Function“DealWithQuery” is responsible for dealing with a query. For N0, he generates the query q and its requirement rq. If Ni receives Emsgi, he decrypts it with his own secret key Si. If q finished its obfuscation phase at Ni, q will be required to be forwarded in free phase immediately. Otherwise, q needs to be processed in the obfuscation process. Both q and rq are stored in Ni until they are sent to the next friend. Ni starts detecting friends continuously if and only if Ni carries one or more obfuscation phase queries.

When Ni detects a new neighbor Nx (one-hop or multi-hop neighbor), he follows steps in “WhenEncounterUser”. For an expired query, Ni simply drops it. If the query q is already inside its obfuscation area, Ni switches it to the free phase. If q stays in the obfuscation phase and Nx is an available friend, Ni encrypts both q and rq using Nx’s public key Kx to get an encrypted message Emsgx. Then Ni forwards Emsgx to Nx and stops sensing friends after Emsgx departs from it.

When we mention that Ni switches a query q to the free phase, Ni actually replaces N0’s information with its own one in q to get q* and records this replacement in its storage, then Ni uses the Spray and Wait protocol [19] to forward q* in plaintext. That allows Ni to hide N0’s identity and forward a reply from LBSP to N0.

Requirement parameters

In the obfuscation phase, rq is always in msg so that friends who get msg can make decisions (e.g. selections of friends) based on it.

All parameters (i.e., Sid, Did, Rp, Rs, Ls ,Tmin and Cmax) in rq are given by N0 before Emsg1 leaves N0. Parameters Sid and Did record the identities of N0 and the destination (LBSP) Nd, based on which last friend Nf is able to send the query freely to Did  (Nd) and forward the reply to Sid (N0).

The obfuscation area is a ring with an inner radius Rp and an external radius Rs. As shown in Fig. 1a, the obfuscation area is actually the grey area “a”. Obfuscation area must guarantee both the original requester’s location-privacy and location awareness. In other words, the value of Rp should be big enough, so that there are sufficient users in the inner circle (with a radius Rp). At the same time, Rs should be small enough so that the LBSP can provide a service, acceptable to N0.

A user Nx can be chosen by Ni as a friend for q if and only if SVi,x is bigger than the threshold Tmin. The original requester N0 can set various values for his queries based on their importance. If Tmin is large, there would be fewer friends for any users in the network which reduces the query success rate to a certain extent, as a result. We assume that the original requester can balance the level of privacy and the success ratio.

Most DTN routing protocols aim to deliver queries through the shortest path, while MHLPP pays more attention to security in its obfuscation phase. Consequently, the obfuscation process in MHLPP results in a longer path from the original requester N0 to the destination Nd. To limit the length of the path, we introduce parameter Cmax which is the maximum extra path (e.g., the difference of the length and the distance between the Ni and LBSP) we can tolerate. For any friend Ni who gets a Cmax from rq, if he selects Nx as the next friend, the extra path should not be longer than Cmax. Let’s denote the optimal path from user m to n by Dis(m,n), and the extra path from Ni to Nd through Nx by Ci,x,d. Then Ci,x,d  can be defined as follow.

(1)

Ci,x,d must be a value smaller than Cmax. If Dis(m,n) is the straight-line distance between point m and n, then next friend Nx should be in an ellipse EC with focus points Ni and Nd. Let’s denote the coordinate of Ni by  and the coordinate of Nd by . Then, the equation of the ellipse EC is

(2)

As shown in Fig. 1a, Ls is the center of the ring while Rp and Rs are the inner and external radii, respectively. The query q switches to the free phase when it enters the ring area.

As shown in Fig. 1b, Ni who is carrying obfuscation queries should choose his next friend NxNx in the ellipse, which avoids the query q going through an unacceptably long path.

In conclusion, a query q starts at the center and moves inside the ring, until it reaches the obfuscation area. The point Ni in Fig. 1b should be inside the ring, and the point Nd might be anywhere. As a result, a user Ni who is carrying an obfuscation query detects a friend continuously who has a larger distance from LS and inside an ellipse. If there is a friend like that, Ni sends the query to that friend Nx.

Privacy Analysis

We assume that attackers can achieve all information in LBSPs know. Obviously they can know the identity of the last friend Nf who replaces N0’s information with his own. It is possible for the attackers to locate Nf with little cost. For example, if  N0 stops moving after sending the query q, it is reasonable for Nf to believe that N0 is in a ring centered at the location of itself with radii Rp and Rs.  In other words, the distance between Nf and N0 should be in a range between Rp and Rs. If attackers find all users who satisfy this condition, the original requester might be among these users with high probability. Then, a success ratio  to locate N0 can be measured by a conditional probability

(3)

where Prprs is the probability that N0 is in the ring (i.e., the distance between N0 and Nf is larger than rp and smaller than rs). Here, mrprs is the number of users who are in the ring. Attackers locate N0 successfully if and only if N0 is in the ring, at the same time, attackers pick the correct one from all  users at that area.

In the worst case, attackers know exact values Rp Rsand RsRp. Then, the Eqn. (3) becomes

(4)

where Prprs is the probability that N0 is on the ring (i.e., the distance between N0 and Nf is larger than Rp and smaller than Rs). mrprs is the number of users who are in the ring. Since parameters (e.g., Rp and Rs) in rq are kept secret among trusted friends in our system model, attackers can hardly get the actual values of those parameters.

Complexity discussion

In order to guarantee secure communications among friends, encryption is introduced in our protocol. In the obfuscation phase, the query is transmitted along friends, i.e. N0, N1, N2, ..., Nf. When the query is sent from Ni to Ni+1, a pair of encryption and decryption is needed, so the number of such pairs Ten should be equal to f.

Ten grows with both Tmin (threshold used to decide friend relationship) and inner radius Rp. Essentially, it is the number of friends participating in transmitting a query q in its obfuscation phase that influences Ten. Given a smaller Tmin, a user carrying the obfuscation phase query has more chances to encounter more friends in a certain area. A larger Rp also leads to a bigger area inside the ring, so that there are more friends in this area. We evaluate the number of encryptions and decryptions in our simulation. Fig. 2 shows the average number of encryptions (Ten) with different friend thresholds Tmin and inner radius Rp. We observe that Ten increases steadily as we increase Tmin and Rp.

It is evident that encryption and decryption process in MHLPP results in an extra cost in both energy and computational resources. However, the number of encryptions and decryptions is quite low (below 3) which reasonable based on the experiment.

Performance Analysis

We use the map of Helsinki in our simulator to evaluate MHLPP. It is also compared against the known protocol,  Hybrid and Social-aware Location-Privacy in Opportunistic mobile social networks (HSLPO) [5]. The simulation parameters are shown in Table II. All pedestrians and cars are users in MHLPP. These users are moving on the map along streets continuously. There is an LBSP fixed at a random location on the map. For each user, we give him random social values between 0% and 100%, each corresponding to all other users. Each value has the same probability, so we can compute the expected number of friends of an user. For example, if we are given a privacy threshold (Tmin) of 85%, then there might be 15% (100%-85%) users who are friends of a certain user.

As shown in Table II, there are 126 users in the map. For each of them, say user i, we give him 126 random SV values, which denotes the relationship strength between him and other users, so that there are 1262 SVs in our simulation. The SVs are between 0 and 100. As a result, if the Tmin is equal to 85, the average number of friends of each user should be 18.9 (=).

| Parameter | Value |
| --- | --- |
| Simulation Time | 10 minutes |
| Map Size (W x H) | 4500 m x 3400 m |
| Total number of users | 126 |
| Pedestrians/ Cars | 84/42 |
| Communication Area Radius | 10m – 90 m |
| Pedestrian Speed | 1.8-5.4 Km/h |
| Car Speed | 10-50 Km/h |

Users are placed at random locations at the beginning of each experiment. We choose another random point for each user, so that he can move back and forth along streets between that point and the point his starts at. The user speed depends on its type (pedestrians or cars) and set randomly. All queries have a 10-minute timeout. The queries which are expired before they reach the LBSP (the destination) are considered to be failed in our success ratio statistics.

Figs. 3-5 compare performances between HSLPO and MHLPP for different values of kk, communication radius and privacy threshold (Tmin in MHLPP). The k is the privacy-level requirement in HSLPO. Both HSLPO and MHLPP have different criteria in which a query can switch to the free phase. To make them comparable, we create a new parameter, called obfuscation distance. If a query leaves N0N0 at location LaLa and switches to the free phase at NfNf whose location is LbLb, then the obfuscation distance is the straight-line distance between La Laand Lb. We test the obfuscation distances of HSLPO with different parameters, and then we set the inner radius of MHLPP to those values. The query success ratio is the ratio of delivered queries to the total number of queries. The number of hops (h) is the number of intermediate users between N0N0 and the destination (LBSP). We count the number of users surrounding the last friend in a specific range, which is kk times the communication radius. We calculate the entropy using the reciprocal of the number of surrounding users.

Query success ratio

The query success ratio is the percentage of delivered queries among a number of attempts. Based on the timeout value in Table II, a query is delivered successfully, if it arrives at the LBSP (the destination) before the timeout; otherwise it fails. We use the query success ratio to evaluate the delivery performance of MHLPP.

(a)

(b)

(c)

Fig. 3.  The query success ratio comparison between HSLPO and MHLPP. (a) Query success ratio with various K. (b) Query success ratio with various communication radiuses. (c) Query success ratio with various privacy thresholds.

As shown in Fig. 3a, the success ratio in MHLPP is always higher than that in HSLPO. As the value of kk increases, HSLPO success ratio drops sharply while MHLPP remains stable. This is because the larger k is, the harder it is for HSLPO to find enough friends in a limited time. The lack of friends has less impact on MHLPP. We observe that the success ratio of MHLPP rises when k = 7k=7. That is because it depends on the inner radius which is equal to the obfuscation distance of HSLPO. The obfuscation distance decreases when k = 7k=7, because most of the queries which complete their obfuscation phase have a short obfuscation distance. In Fig. 3b, both HSLPO and MSLPP values increase and have the same trend when given a larger communication radius. The reason is that the communication radius effects the free phase more than the obfuscation phase for both. As shown in Fig. 3c, higher privacy threshold leads to lower success ratio in two algorithms. Its impact on HSLPO is more intense than that on MHLPP, which is the most important characteristic of MHLPP. MHLPP has a better performance than HSLPO especially when there are fewer friends in the network. MHLPP can transmit messages with the help from strangers in its obfuscation phase while HSLPO cannot.

Number of Hops

We count the number of hops it takes for queries to be delivered successfully and calculate the average. Every user who takes part in the delivery process is considered in the hop count. We introduce this criterion to measure the routing path length of the algorithm. MHLPP is more sensitive to the probability that a user encounters a friend than HSLPO is. The reason is that MHLPP aims to reach a certain distance rather than taking certain number of hops. In other words, MHLPP continues sending queries to other friends until queries enter their obfuscation area. In this process, MHLPP takes every chance to forward queries. If it is hard for MHLPP to find friends, it can also take the queries to obfuscation areas with fewer friends. Therefore, the probability that users encounter friends has less impact on the performance of MHLPP. The result of this experiment is shown in Fig. 4.

(a)

(b)

(c)

Fig. 4.  The number of hops comparison between HSLPO and MHLPP. (a) number of hops with various k. (b) number of hops with various communication radii. (c) number of hops with various privacy thresholds

In Fig. 4a, the number of hops in HSLPO is affected by parameter k obviously. Especially, the first k hops forwarders  must be friends, which makes it hard for HSLPO to have  queries to be forwarded  successfully. Both protocols have similar number of hops in their free phases, but HSLPO has exactly k hops in its obfuscation phase, while MHLPP can have fewer than k hops. In figure 4b, the number of hops in MHLPP grows with the communication radius obviously, while it does not change a lot in  HSLPO, because only successful delivery queries are counted in the statistics. Given a large communication radius, MHLPP has a much higher success ratio for delivered queries as it can connect more friends. In Fig. 4c, the value of MHLPP drops for higher privacy thresholds. The reason is the same as Fig. 4b. For HSLPO, no matter how hard it is to find a friend, it attempts to find exactly k friends. However, if it is too hard to find a friend, MHLPP’s friend can carry the query while moving and complete the obfuscation process. For higher privacy thresholds (i.e. resulting in fewer friends), MHLPP chooses to carry queries other than finding friends. That results in a drop in the number of hops.

Security

Since the principles with that two protocols protect original requesters are different, we evaluate the probability that attackers locate the original requester if the distance between him and the last friend is smaller than a some value r, which is equal to k times the communication radius. Since the last friend reveals himself to the LBSP, attackers might locate him accurately. We assume that attackers know the privacy parameter k. In the worst case, the distance between the original requester and the last friend is smaller than r when attackers start to locate the original requester. That gives attackers a chance to locate the original requester. We count all users who are inside the radius r of the last friend, and the original requester is one of them. For example, if there are m users in the area, the probability should be 1/m. Figure 5 compares the entropy E of both HSLPO and MHLPP. We use the following formula for computing entropy:

E=-1mlog21m

From Fig. 5a, we observe that MHLPP  has very small (about 0.04) increase in entropy compared to HSLPO. When the original requester is in the circle centered at the last friend, HSLPO is a little more secure than MHLPP but not by very much.  That is because HSLPO always switches to the free phase when the last friend encounters the previous friend, so the previous friend must in the circle. MHLPP does not have this condition. Both graphs in Fig. 5a and Fig. 5b have the same trend. The curve of MHLPP is also a little higher than that of HSLPO, while two curves almost meet when the communication radius is small or large. Given a small radius, the entropy of both protocols are small. When the radius is large, we can ignore the effect of the previous friend mentioned about for Fig. 5a as there are so many users in the circle. From Fig. 5c, since the circle neither expands or shrinks, we observe that the two curves exhibit similar behavior. The values of HSLPO are always lower than the correlated values of MHLPP. However, as we observed in the experiment, the last friend is always hundreds of meters away from the requester.

(a)

(b)

(c)

Fig. 5.  Locating probability entropy comparison between HSLPO and MHLPSP. (a) Locating probability entropy with various K. (b) Locating probability entropy with various communication radiuses. (c) Locating probability entropy with various privacy thresholds.

Appointment Card Protocol

System Model

The network architecture consists of two main entities: Users and Location-Based Service Providers (LBSPs). Users are mobile and communicate with others in a certain range, e.g., the communication range of their portable devices. For a user, other ones in the social network are either strangers or his friends whom he can detect when they are in his communication range. Let RSi,j denotes the relationship strength between user  i and j, if RSi,j is larger than a specific friend threshold FTmin, user j is a friend of user i. LBSPs are fixed and not connected in a social network, which provides Location-Based Services (LBSs) for users. We assume that the only information which is necessary for the LBS is a location from the original requester, but the original requester should still give an identity to the LBSP so that the LBSP can reply to that identity.

We consider only an external attacker capable of eavesdropping on limited traffic in the network. We assume that the attacker can access the database of LBSPs, so that he can learn everything recorded in LBSPs, including identities and locations. He launches an inference attack to each user who use the LBS and learns more privacy information based on the information including the location and the context in the queries. Therefore, the key of protecting location-privacy is degrading the relationship between his identity and the location provided by him so that the attacker can hardly infer the identity of the original requester by the known information.

We propose an Appointment Card Protocol (ACP) to protect the identity and location-privacy of the original requester by providing other users’ identity (agents), which can be any user in the network, so that ACP provides large anonymity set. The friends of the original requester separate the agents and the original requester, so that the agents have no knowledge about the original requester.

Appointment Card Protocol Overview

We propose an Appointment Card Protocol (ACP) to protect original requesters when they are served by LBSPs. A user (Agt1) generates his own Appointment Cards (ACs) containing his own identity called Cid and a unique number called Capt(Creator Appointment number). The appointment cards are exchanged when two users encounter each other. When the original requester sends a query, he chooses an AC and sends the query using the identity Agt1 which is in the AC. A LBSP replies to Agt1 when it receives the query. Agt1 is the one who generates the AC and the first agent of the AC. He re-transmits it to the next agent (Agc2) whom he gives the AC to, and so do the following agents until the reply reaches the last agent. The last agent is responsible to forward it to the original requester.

Figure 4.1 Whole Process Example

The Figure 4.1 is an example of the whole protocol, the explanation of the symbols in the figure is shown in Table 4.1. These symbols and the figure are used in the whole chapter to help us describe the protocol. To make it clean, we may omit some superscript and subscript from those symbols in the following sections when there is no ambiguity. The whole process can be considered as the following parts: 1). exchanging cards among all users who are called agents (i.e., 1 and 2); 2). exchanging cards among friends (i.e., 3); 3). sending query using information of appointment cards (i.e., 4); 4). forwarding the reply among agents (i.e., 5, 6

Table 4.1 Symbols

| entry | explanation |  |
| --- | --- | --- |
| Uε | A user whose identity is ε. |  |
| ACαβ | The βth appointment card generated by a user α. |
| ACα,γβ | ACαβ is being forwarded by an agent γ. |
| Queryδαβ | A query whose original requester is δ and using ACαβ |
| Replyαβ | The reply of a query which uses ACαβ. |
| Replyγαβ | Replyαβ is being forwarded by an agent γ. |
| Agtiαβ | The ith (i≥1) agent of ACαβ. |
| captαβ | The parameter Capt in ACαβ. see Table 4.2 |
| aaptγαβ | The parameter Aapt in ACαβ, which is given by an agent γ. see Table 4.2 |
| AN | Both the Capt and the Aapt in an AC are called the Appointment Number. |

and 7); 5). relay to the original requester (i.e., 8).

Appointment Card

Since the original requester cannot use his own identity to communicate with the LBS provider (LBSP), he must use others’ identity (Agt1) to send queries, so that the LBSP can reply to the original requester through Agt1. Appointment cards make it possible that agents forward the reply to the original requester one by one. In other words, the appointment card indicates a

Table 4.2 Appointment Card

| entry | explanation |  |
| --- | --- | --- |
| Cid | The identity of the creator who generates the AC. |
| Capt | A unique number that distinguishes an AC from other ones generated by the same creator. |
| Aid | The identity of an agent who gives the AC to the recent holder. (The previous hop of the AC) |
| Aapt | A unique number that distinguishes an AC from other ones transmitted by the same agent. |
| time-out | The time when the AC expires. |
| EQ | A queue (Exchange Queue) which records users who exchange the AC in order. Its length is EQL. |

path, through which U0 can get its reply.

In Figure 4.1, the user a, b and c are the agents of the appointment card ACaβ (i.e., Agt1aβ, Agt2aβand Agt3aβ). These agents are strangers, so that attackers can hardly infer c from the identity a. At the same time, c is in the original requester d’s social tie (i.e.,c is d’s friend or his friends’ friend, ...), and he is the only one who knows how to reach d. Therefore, it is hard for attackers to infer d from a.

Notice that c receives a’s appointment card (i.e. ACaβ) from a stranger b who knows the information of ACaβ and the identity of the next agent c, so that it is unsafe for c to use that appointment card. In other words, the appointment card cannot be used until c exchanges it to another user (e.g., the user d) who trust c. The appointment card is called the ready appointment card (ready AC) after it leaves the last agent (i.e., the user c), or it is called the distributing appointment card (distributing AC). It is obvious that distributing ACs are transmitted among agents who can be strangers, while a user can only get ready ACs from a friend of him.

To make users carry a similar number of ready ACs, ready ACs are also exchanged between friends. As a result, the last agent does not sure whether the user whom gets the ready AC from him is the original requester. We introduce a pseudonym mechanism, which enables the last agent to forward the reply to an unknown original requester.

All users in the network are responsible to generate their ACs respectively and they are called the creators of their own ACs. The creator writes down his own identity (Cid), a unique Creator Appointment Number (Capt) on his AC. When users exchange ACs, they modify Aid (the Agency ID) and Aapt (the Agency appointment number) in the AC to enable the next agent to identify whom is the previous one. Entries of AC are shown in Table 4.2.

AC life cycle

Figure 4.2 AC's life cycle

The life cycle of an AC starts when it is generated by its creator. The first k (see Table 4.3) agents add their identities into its EQ (see Table 4.2) before exchanges it, which increase the length (EQL) of the EQ. When the AC’s EQL reaches k, it is eligible to be used in a query and called a ready AC. When an AC is used in a query, it is marked as an used AC by the original requester who uses the AC. No matter what state (distributing, ready or used) an AC is in, it can expire, as shown in the Figure 4.2. An AC starts at the distributing state. If the length of its EQ reaches k, it is switched to the ready state. It can timeout in all states. If an AC is used for once, it is switched to the used state. A used AC can also be used in other queries but cannot be given to any. All the system parameters are shown in Table 4.3. The purpose of introducing these system parameters is the following sections. After that, we

Table 4.3 Important system parameters

| Parameters | explanation |  |
| --- | --- | --- |
| k | The obfuscation distance |
| m | The friend obfuscation distance |
| Seg | The distributing segment |
| GP | The generating period of appointment cards |
| AT | The timeout for appointment cards |
| τ | Avoiding time |

introduce the whole process of the ACP.

System parameters

Obfuscation distance

The obfuscation distance k is the number of exchange before an AC is switched to the ready state. In other words, an AC must be exchanged for k times before it becomes a ready AC.

Figure 4.3 obfuscation distance

As shown in Figure 4.3, an AC is exchanged along Agt1, Agt2, ..., Agtk. Since those agents are strangers, the only relationship between two adjacent agents is that they encounter each other somewhere. The relationship between Agt1 and Agtk becomes weaker when we increase k. In other words, attackers can hardly infer the identity of Agtk when he only knows the identity of Agt1, and his difficulty increases with the parameter k. As a result, it is hard for attackers to infer the original requester, even though Agtk is in the social tie of the original requester. Since the reply message must go along the serial of the agents, a long obfuscation distance also lengthens the path of the reply message. Therefore, a large k makes the original requester safer, while making it harder for the reply longer to be delivered.

Friends obfuscation distance

Figure 4.4 Friends-obfuscation distance

Since Agtk-1 is a stranger for Agtk, it is possible that Agtk-1 is exactly the attacker. We assume that the attacker knows that Agtk is in the social tie of the original requester. The attacker can assume that Agtk is a close friend of the original requester, so the identity of Agtk gives the attacker a good tip to infer the original requester. A solution to prevent the agent Agtk-1 from learning the original requester easily is that the last m agents are friends, as shown in Figure 4.4. The m is called friends obfuscation distance, and the last m agents are called trusted agents.

It is true that Agti is a friend of Agti+1, i>k-m, but two nonadjacent trusted agents (e.g., Agti and Agti+2) might have a weak relationship. Since there are at least m trusted agents between the stranger Agtk-m and the original requester, Agtk-m can hardly infer the original requester based on information he learns. When m=k, all ACs must be exchanged between friends only. When m=1, the last agent is the only one who is in the social tie of the original requester, which is the case in our example.

However, friends encounter each other rarely so that a big m increases the difficulty of distributing ACs. In this paper, we assign m to 1.

Distributing segment

We also use the example in Figure 4.1. If Ua generates 3 ACs (i.e. ACa1, ACa2 and ACa3). It exchanges these ACs to Ub, then Ub exchanges them to Uc and so on. At last, all of them reach the original requester Ud. In this case, Ud has several ACs whose routes (the list of agents) are the same, so that Ud has no choice. In other words, since using these ACs results in the same reply routes, Ud is restricted to choose an optimal AC based on specific situation.

If agents exchange ACs to different users, the above problem will not happen. We use a system parameter distributing segment Seg to avoid giving all ACs which are received together to the same user. Each user has Seg Distributing AC Lists (DLs). It put received distributing ACs in one of those DLs randomly. If Agti exchanges ACs with another user, Agti selects one of his DLs, and only ACs in that DL will be exchanged to the other user.

For example, we assign Seg=2 so that each user has two DLs (i.e., DL1 and DL2). When Ub receives ACa,a1, ACa,a2 and ACa,a3 from Ua, Ub may put ACa,a1 and ACa,a2 in its DL1, while ACa,a3 in its DL2. If Ub encounters Uc, Ub can give either ACa,b1 and ACa,b2 or only ACa,b3 to Uc. In this way, we separate ACs generated by the same creator.

Generating Period

Since ACs could expire, users must generate new ACs continuously. We use GP to denote the speed of generating new ACs per user, that is, each user generates a new AC every GP second.

AC timeout

Let AT denote the timeout of ACs. An AC expires after it is generated for AT seconds. When an AC expires, all agents delete its information from their memory.

Avoiding time

If a user gets an AC at time ti, he cannot get that AC again before ti+τ, where τ is parameter avoiding time. It is described in 4.6.2.

Protocol Details

Generating appointment cards

Maintaining a certain number of ACs in the network is a prerequisite for users to sending queries. Considering that ACs can expire, users must generate ACs continuously. The user who generates an AC is called the creator of the AC, at the same time, he is also the first agent of the AC. ACs are generated based on 2 principles, which are fairness and continuity.

ACs impose burdens on agents. In fact, agents are unlikely to benefit from relaying messages to the original requester. Because they need to allocate memories to save ACs’ information and cost more energy for forwarding replies. To avoid a part of users who generates more ACs than others going into overload, fairness is an essential part of the generating strategy. In other words, users should generate a similar amount of ACs.

We assume that the AC timeout (i.e. AT) is 30 minutes, and every user generates 100 ACs at the beginning (e.g., the 0th minute) and generates no AC until the 30th minute. Since all agents remove the information of expired ACs from their memory, a reply can hardly be delivered when the AC it uses is timeout. As a result, it is hard for a user to select an appropriate AC for his query at the 29th minute when all ACs only have 1 (=30-29) more minute, because his reply may lose even though his query is delivered successfully. Therefore, the generating strategy should be a steady and sustainable, instead of an intensive one.

In our protocol, each user generates a new AC every GP second. Since an AC has a AT-seconds timeout, there are about δ=ATGP appointment cards hold by each user in the network. In other words, each user maintains about δ ACs which is generated by himself, and we achieve the fairness part. Since users generate ACs continuously, ACs have various timeout so that it is likely for a user to pick an AC who does not expire in a long time (at least longer than his queries and replies).

Exchange distributing appointment cards

Users exchange their distributing ACs as frequently as possible, so that a distributing AC can be switched to a ready one quickly. Still, there are some other conditions which should be obeyed when exchanging a distributing AC.

We assume that two users, e.g. UA and UB, walk together for a long time. If UA generates an AC (e.g., ACA) at t0 and exchanges it to UB, then ACA is given back to A and so on. The distributing phase of the AC only costs a few seconds, while only UA and UB are agents. That loses the meaning of exchanging ACs, because it is easy for an attacker to infer the last agent who may be UA or UB from the first agent UA.

We import an parameter avoiding time τ to optimize the agent selection strategy. If a user gets an AC at time ti, he cannot get that AC again before ti+τ. For example, UB receives a certain AC ACA from UA at t0, then UB sends it to a user C (UC) who can also send it to others. If a user carrying ACA encounters UB  before t0+τ, he cannot send ACA to UB. Therefore, if the parameter τ is larger than or equal to the parameter AT, there is no repeated agent in an AC’s EQ. In other words, an AC never reaches an agent twice with an τ equal to AT. In this chapter, we assign τ to AT.

Besides, we should also avoid ACs having the same sequence of agents, as we mention in the distributing segment subsection. Now we propose the strategy of exchanging distributing ACs.

Let us take a pair of users Alice and Bob as an example. If Alice encounters another user Bob, Bob tells Alice whether he trusts her (She is viewed as a friend by Bob). Alice picks one of her distributing AC lists (e.g. DL1). Alice traverses all ACs in DL1, and distributing ACs which match all the following two conditions are exchanged to Bob.

- If the length of the AC’s EQ (i.e. EQL) is not shorter than k-m, then Alice must be a friend of Bob.

- Bob does not carry the AC in the recent τ (time).

When Alice is sending a distributing AC to Bob, she adds her identity and recent time to the AC’s EQ. Besides, Alice must modify the AC’s Aid to her own identity and its Aapt to a new one. The information in Table 4.4 are recorded in her memory where is called the relay table. We should notice that the first agent (i.e., the creator) does not have a Aaptold, because he is exactly the one who generates the AC, then the Aaptold should be his own identity. When Bob gets those ACs, he puts each one of the received distributing ACs to his DLs respectively and randomly. If an AC whose EQL is already equal to k, the AC must be switched to a ready one when Bob gets it. Bob puts ready ACs in his ready AC list instead of distributing AC lists.

Table 4.4 relay table entries

| Name of entries | descriptions |
| --- | --- |
| Aaptold | The Aapt generated by the previous agent |
| Aidold | The identity of the previous agent |
| Aaptnew | The new Aapt (generated by himself) |
| IDnxt | The identity of the next agent |
| EQL | The length of the AC’s EQ (should larger than 0) |
| AT | The time when the AC timeout. |

Exchange ready appointment cards

Users asks for Ready ACs only from their friends, which ensures that the information of the ready ACs hold by a user is not exposed to strangers. The strategy of exchanging ready ACs faces a problem. That is, the number of each user’s ready ACs should be similar.

We also take friends Alice and Bob as an example. Alice has 20 ACs, while Bob has 10 ACs. When they encounter each other, it is reasonable that they both gives half of their ready ACs to the other one. As a result, they both will have half of total (15) ACs. However, that strategy does not work all the time.

The problem becomes more complex in the following condition. Alice is a friend of Bob, while Bob is not a friend of Alice. In other words, Bob trusts Alice, but Alice does not trust Bob. We assume that Alice is a trustful but suspicious girl, so that many users trust her while she trusts few users while Bob is opposite. When users encounter Alice, they ask for ready ACs from her, but Alice rarely asks for ready ACs. Therefore, Alice carries few ready ACs, while Bob carries so many ready ACs. When Alice and Bob encounter each other, it is Bob who is asking for ready ACs instead of Alice. To make the strategy fair and efficient, users must compare the number of their own ready ACs and the number of ready ACs carrying by the other user when two users are exchanging ready ACs.

When Bob encounters Alice, Bob tells Alice the number of his ready ACs (NRBob) and whether he wants Alice’s ready ACs (if he trusts Alice). If Alice learns that Bob needs her ready ACs, she compares the number of her ready ACs (NRAlice) and NRBob. If NRBob≥NRAlice, Alice gives no ready AC to Bob; otherwise, she gives NRAlice-NRBob2 to Bob. In this way, ready ACs do not concentrate in a group of users who trust many users.

The process of exchanging ready ACs is much simpler than that for distributing ACs. Users do not modify any information in the ready ACs including Aapt and Aid, so that ready ACs keep static after they leave the last (kth) agent.

Sending queries

For an original requester who want to send queries to the LBSP, his query must be delivered to the LBSP while the LBSP cannot learn the identity of him. Besides, the reply message from the LBSP must be delivered back to the original requester. The basic idea is that the original requester sends his query using another user’s identity to the LBSP, so that LBSP can reply to that user who is responsible to forward the reply to the original requester. That user is the first agent (Agt1) of the AC which is used by the queries of the original requester.

In order to enable LBSP to reply to Agt1, the original requester’s query includes an sender identity Agt1, which is equal to the Cid in the AC. Since Agt1 needs a Capt to identify the AC used by the query (and the reply), the Capt in the AC is also in the query. The network can deliver that query to the destination LBSP easily with any DTN protocols.

We use the example in Figure 4.1. The original requester Ud has an AC (i.e., ACa,cβ) whose creator is Ua. When Ud uses it to send his query Querydaβ, the sender identity is a and the Capt of the query is the Capt of ACa,cβ, as shown in Figure 4.5

Figure 4.5 constitute query

The AC is marked as used when the query is ready to be sent, so that the AC cannot be exchanged to other users. To get the reply, the original requester also uses a pseudonym, which is described later.

Sending replies

The LBSP part

When the LBSP received the query, it learns that the sender’s identity is the first agent instead of the original requester, which protects the location privacy of the original requester. The LBSP reply to the first agent using the information in the query.

Figure 4.6 constitute replies

In Figure 4.6, the LBS provider reply to the sender Ua, when it receives Querydaβ. The Capt of the query is also included in the reply, which enables Ua to identify the AC used in the query and the reply.

The first agent

When the first agent (i.e. Agt1) gets the reply from the LBSP, he learns the AN in the reply. He searches his reply table to get the information of the AC used in the query and reply. If the AC does not expire, there must be a corresponding entry in his reply table, as shown in Table 4.5. Agt1 gets an entry where the Aidold and Aaptold are equal to his identity and the AN of the reply. As a result, he learns the identity of the next agent (i.e. Agt2) from the IDnxt of the entry, so Agt1 forwards the reply to Agt2. The AN of the reply is replaced with the Aaptnew in the entry by Agt1, which enables Agc2 to identify the AC in Agt2’s reply table.

Table 4.5 reply table entries of the first agent

| Name of entries | value |
| --- | --- |
| Aidold | The user’s own identity |
| Aaptold | The Capt of the AC used in the reply (query) |
| IDnxt | The identity of the second agent |
| Aaptnew | The Aapt given to the second agent by the user. |
| EQL | 1 |
| AT | The time when the AC timeout. |

For the example in Figure 4.1, the first agent is Ua. When he receives ReplyLBSaβ, he learns that it is a reply from the LBS and the Capt of the AC is captaβ. He searches his reply table for an entry whose Aidold is equal to a and Aaptold is equal to captaβ. As shown in Figure 4.7, the IDnxt in the entry is equal to b so he modifies the receiver of the reply message to Ub. Ua also modifies the AN in the reply message to aaptaaβ which enables Ub identifies the AC in his reply table, because the Aaptnew in the entry is equal to aaptaaβ.

Figure 4.7 the reply of the first agent

Intermediate agents

The process of forwarding replies in the intermediate agents (the second to the k-1th one) is similar with that of the first agent. We take the second agent as an example. When the second agent receives the reply forwarded by the first one, he learns the sender’s identity (i.e. the first agent) and the AN from the reply. Since he gets the AC which is used in the reply (query) from the previous agent, there must be an entry, whose Aidold is equal to the previous agent’s identity and the Aaptold is the value of Aapt in the AC when he gets it, in his reply table, as shown in Table 4.6.

Table 4.6 reply table entries of the second agent

| Name of entries | value |
| --- | --- |
| Aidold | The identity of the previous agent (i.e. the first agent) |
| Aaptold | The Aapt given by the previous agent. |
| IDnxt | The identity of the next agent (e.g. the third agent) |
| Aaptnew | The Aapt given to the next agent by the user. |
| EQL | 2 |
| AT | The time when the AC timeout. |

For the example in Figure 4.1, the second agent is Ub. When he receives Replyaaβ, he learns that it is Ua who forwards the reply message Replyaaβ and the Aapt of the AC is aaptaaβ. He searches his reply table for an entry whose Aidold is equal to a and Aaptold is equal to aaptaaβ. As shown in Figure 4.8, since the IDnxt in the entry is equal to c, he modifies the receiver of the reply message to Uc. Ub also modifies the AN in the reply message to aaptbaβ which enables Uc identifies the AC in his reply table, because the Aaptnew in the entry is equal to aaptbaβ.

Figure 4.8 the reply of the second agent

For each agent Agti, where 2≤i≤k-1, he searches his reply table for a correlative entry, when he receives a reply. The Aidold and Aaptold in the entry should be equal to the sender’s identity and the AN in the reply. The identity of the next agent Agti+1 is IDnxt. Agci also assign Aaptnew to the AN in the reply to help Agti+1 search Agti+1’s reply table.

The last agent

The last agent also searches for a reply table entry based on the reply, while he cannot get the identity of the next agent. The reply table entry of the last agent is shown in Table 4.7.

Table 4.7 reply table entries of the last agent

| Name of entries | value |
| --- | --- |
| Aidold | The identity of the previous agent (i.e. the first agent) |
| Aaptold | The Aapt given by the previous agent. |
| IDnxt | VOID |
| Aaptnew | The Aapt given to the original requester. |
| EQL | k |
| AT | The time when the AC timeout. |

The last agent uses the same way to look up an entry in his reply table as the previous agents. When he finds that the EQL is equal to k, he notices that he is the last agent. Then he is responsible to forward the reply to the original requester instead of another agent. He uses his identity and Aaptnew in his reply table entry to generates a pseudonym pseudoAgck,Aaptnew, where the function pseudoid,Aapt is a public pseudonym generating function which everyone in the network knows, including the original requester.

When the original requester sends his query, he also gets the same pseudonym pseudoAgck,Aaptnew using the pseudonym generating function. Note that he can get parameters from the AC. Agck and Aaptnew are equal to the Aid and Aapt in the AC. He uses that pseudonym as his identity before he gets the reply.

When users deliver the reply from the last agent, they are looking for a user whose identity is that pseudonym. At last, the original requester gets the reply, because he is the only user who uses the pseudonym as his identity.

For the example in Figure 4.1, the last agent is Uc. When he receives Replybaβ, he learns that it is Ub who forwards the reply message Replybaβ and the Aapt of the AC is aaptbaβ. He searches his reply table for an entry whose Aidold is equal to b and Aaptold is equal to aaptbaβ. Since the EQL in the entry is equal to 3 (i.e. k), he recognizes that he is the last agent. Uc calculates the pseudonym psdcaβ=pseudoc,aaptcaβ then forward the reply to psdcaβ. The original requester d gets the identity c and the aaptcaβ from the AC he uses, so that he uses the pseudonym psdcaβ as his identity. As a result, Ud get the reply from Uc. The AN of the reply is also assigned to aaptcaβ to avoid identical pseudonyms. The process is shown in Figure 4.8.

Appointment number

The Appointment Number (AN) including Capt and Aapt is significant information in the AC. We explain the rules of generating them in detail and talk about the effect of ACs’ timeout mechanism in this section.

Creator appointment number

The Creator Appointment Number (Capt) is a number which is used to identify ACs generated by the same creator. In other words, if two ACs are generated by the same creator and they do not expire, their Capt must be different. Therefore, the first agent (i.e., the creator) cannot find two entries which have the same Aaptold in his reply table, if their Aidold are both his own identity.

Agent appointment number

The Agent appointment number (Aapt) is a number used to identify appointment cards who have the same agent. Agents generate a new Aapts for appointment cards before they exchange those cards to others. In other words, an agent gives any appointment card passing on by him a unique Aapt, which helps the next agent identify the appointment card in his reply table. Since the Aapt is unique, an agent who is not the first one cannot find two entries which have the same pair of Aidold and Aaptold, neither.

Timeout

Agents delete the entries, which contains the information of expired ACs, from his reply table. Consequently, agents cannot forward replies using ACs which expire before this very moment. That is the reason why an original requester must choose a AC which expires after his query and reply timeout. While the timeout mechanism has more advantage than disadvantage.

Since users are moving, the distance between agents and the original requester might be too large after a long time. As a result, it is hard for agents to forward the reply messages back to the original requester, so that the original requester takes a higher risk when he uses an AC which is generated for a long time. Then these kind of ACs might not be used after a period, while it cost agents a few memories to save the ACs’ information in their reply table. Therefore, all users remove the information of expired ACs to save their memories.

We should also notice that an unexpired AC and an expired one might have the same Capt or Aapt. Because no agent keeps the record of the expired AC, so that the duplication cannot confuse agents.

Experiment

We use the map of Helsinki in our simulator to evaluate ACP. It is also compared against Binary Spray and Wait (BSW) [20], distributed social based location privacy protocol (SLPD) [5] and our Multi-Hop Location-Privacy Protection (MHLPP). Users are moving on the map along streets continuously, while there is a LBSP fixed at a random location on the map.

For each user, we give him random social values between 0% and 100%, each corresponding to all other users. Each value has the same probability, so we can compute the expected number of friends of an user. If a user whose social value is larger than 85% is called a friend and there are n users in the network, there are n×85% friends.

The Shortest Path Map-Based Movement (SPMBM) [24] is used in our experiment. For each experiment, we give the simulator a random seed so that it can generate pseudo-random number based on the seed. Therefore, all the factors including users’ speed and locations are the same if two experiments have the same random seed. All those four protocols are tested using the same serial of random seed.

Before each experiment, the simulator runs for 800 seconds (simulator time). Then we pick 100 users out of 126 users randomly, and each of them send a query to the LBSP. Tests last for about 20 minutes (simulator time).

Average query success ratio

The query success ratio is the percentage of delivered queries among a number of attempts. Since users sending 100 queries in each experiment totally, if s queries are delivered to the LBSP at time t, the query success ratio of time t is s%.

Figure 4.10 average query success ratio (10 minutes)

As shown in Figure 4.10, we compare the average query success ratio of the four protocols with 5 kinds of communication radius (10, 30, 50, 70 and 90 meters). We observe that the ACP and the BSW get a high query success ratio, while the MHLPP and the SLPD are lower than the former two protocols. It is obviously that the BSW is the highest one, because it is a no-privacy protocol. The ACP is just a little lower than BSW, because the query delivery process of the ACP is almost the same as that of BSW. Since users of ACP must wait for available appointment cards, they cost more time to initial their queries. But the ACP and the BSW are in the same level, comparing to the other two protocols. The MHLPP and the SLPD need to find friend to obfuscate their queries, which baffles their delivery process.

Figure 4.11 average query success ratio (20 minutes)

The experiment results when we test 20 minutes is shown in Figure 4.11. Comparing to the average query success ratio at 10 minutes, the MHLPP and the SLPD achieve much higher success ratio after 20 minutes than at 10 minutes. That is because it cost them so much time in their obfuscation phases when they need to find friends. In fact, some of queries of the MHLPP and the SLPD still do not finish their obfuscation phase at 20 minutes.

The communication radius can influence the success ratio. In most of cases, the success ratio rises when we increase the communication radius, and its influence is especially obvious under 50 meters. A large communication radius makes it easily for users to encounter others, which is good for them to forward queries. However, a user who is so far away from the destination does not want the intermediates of his query encounters many users nearby. Because all users who carries copies of that query are near the sender instead of the destination, which decreases their query success ratio. Therefore, when the communication radius reaches 70 meters, the success ratios almost stay at the same level.

In the Figure 4.12, we observe that the ACP and the BSW have better convergence speed than the MHLPP and the SLPD. In other words, the former two protocols have a faster speed

Figure 4.12 average query success ratio with 50-meters communication ratio

to approach the 100% query success ratio than the later two. At the very beginning, the ACP even has a little higher success ratio than the BSW. Because the ACP users need ready appointment card, and most of the users get their first ready appointment cards at places where there are many users. Therefore, users rarely generate query near the edges of the map at the beginning, which facilitates their queries delivery process. For example, if a user generates his query at the edge of the map, the copies of his query will be sent to users who is also at the edge and it takes more time for them to deliver the query. If the user does not generate his query until he arrives a place nearer center, the copies of his query will be carried by the users in the center with higher probability.

Average reply success ratio

When the LBSP receives a query, it sends a reply to the requester. If the reply arrives the original requester, we view it as success, otherwise, the reply is failed. Since there are 100 queries in each experiment, the number of replies should be equal to 100. Even though some of queries are not delivered to the LBSP quickly, they will be there if the test last for a long time.

Figure 4.13 average reply success ratio with 50-meters communication ratio

The BSW has a significant and reasonable higher success ratio than all other protocols, because it is a no-privacy protocol. The ACP is higher than the MHLPP and the SLPD, but its advantage is not as large as that in the query process. In fact, the reply process of the MHLPP and the SLPD are simpler than that of the ACP, but the ACP saves so much time in its query process that it earns a better reply success ratio than the other two.

Total number of query relay

The query delivery processes of all the four protocols use the BSW protocol. The BSW makes copies for queries and gives half of copies to any users it encounters. That is a significant cost for the network, so we count the number of forwarding queries to evaluate the cost of the four protocols. For example, in the SLPD, there are two phases: the obfuscation phase and the free phase. In the obfuscation phase, a query is forwarded among one-hop friends for k times. After that, it is forwarded by the BSW protocol. The BSW protocol makes c copies for the query and gives half of the copies to any encountering users. Then the number of forwarding queries should be about k+c. If a user encounters the destination, he gives all its copies to the destination. When the number of his copies is larger than 1, the number of forwarding queries should be smaller than k+c. The smaller that number is, the smaller cost of the network is.

Figure 4.14 average total number of forwarding queries at 20 minutes

In Figure 4.14, we compare the total number of forwarding queries with four protocols. We observe that all the four protocols are at a similar level, the BSW and the ACP is a little lower than the other two. For the ACP and the BSW, they deliver queries so fast that users who carries more than one copies give all their copies to the destination at one time, as a result, many copies have no chance to be forwarded separately. While the MHLPP and the SLPD have obfuscation phases, the queries start to be delivered freely (in a BSW way) at a random place where might be so far way from the destination, so that almost all copies can be forwarded respectively.

The communication radius effects the total number of the forwarding queries, especially for the MHLPP and the SLPD. Those two protocols can finish their obfuscation phase more quickly which a larger communication radius, so that more queries can be forwarded freely (in the BSW way), which makes their total number of forwarding queries larger.

Memory cost

We count the number of queries carried by each user to evaluate the memory cost of the four protocols. Several copies of a query are counted for only once.

Figure 4.15 average query buffer needed

In the Figure 4.15, we compare the number of queries per user with the four protocols at 20 minutes. We observe that the BSW is the highest in most of the cases and the ACP always stays at a similar level as the BSW. The data of other two protocols (the MHLPP and the SLPD) increase as the communication radius. The MHLPP even excesses the BSW when the communication radius is 90 meters. The reason is that quite a number of the BSW and the ACP users forward their copies to the destination so that there is no copy with them at 20 minutes, while the rest of them cannot forward their copies to the destination even given a large communication radius. While the number of the other two protocol’s free phase queries is significantly influenced by the communication radius. The more queries are in the free phase, the more copies are in the network.

The Figure 4.16 shows the average number of queries which are carrying by users when the communication radius is 50 meters. The curves of the ACP and the BSW rise sharply at the beginning and then become flat, while those of the MHLPP and the SLPD rise smoothly and continuously.

Figure 4.16 average number of carried queries per user

Distributing appointment cards

Exchanging appointment cards is a feature of the ACP, which imports burden into the network. We count the number of exchanging appointment cards per minute to evaluate the extra cost of the ACP.

In Figure 4.17, we count the total of exchanging ACs processes in the whole network. For example, if a user Alice encounters another user Bob, the total of exchanging ACs processes increases by one when Alice exchanges any ACs to Bob. We count the number of those exchanging processes occur per minute. As shown in the figure, the exchanging processes do not occur frequently, but about 2 times per minutes. Since the size of an appointment card is a small, it does cost the network many resources. At the same time, users can get many appointment cards to help them send queries, as shown in Figure 4.17. The number of ready ACs per user is raising smoothly and steadily.

Figure 4.17 Average number of ready ACs per user

Figure 4.18 Average number of exchanging ACs per minute

he picks an AC from his ready-list. He gives preference to the AC that has the earliest time-out. As a result, he sorts ACs in the ready-list in ascending order based on their remaining time and selects the first AC (e.g. ac) to send his query. That means the query is sent under the name of ac’s creator (i.e., Cid), and the location of the query is also ac’ location (i.e., Clct). Besides, ac’ Capt is also included in the query. The query can be transmitted to LBSPs by an arbitrary DTN routing protocol (e.g., Spray and Wait). When a LBSP receives the query, it provides services based on ac’s location and reply to ac’s creator (ac’s Cid). Users can also use a general DTN routing protocol to deliver the reply.

The (ready) AC is an authorization that anyone holds it can send a query to LBSPs using its creator’s identity (Cid) and location (Clct). When LBSPs receives the query, it replies to the creator (Cid) and attaches the Capt in reply, which enable the creator to determine who is the next one to transmit the reply.

Both Capt and Aapt are appointment numbers, which are unique random integers. When we mention “unique”, it means that there are not an identical (same value and type) existing appointment number generated by the same user in the network. Existing means that the CA which is relevant to the appointment number does not expires. For example, if a user (Ui) generates a Aapt “123456” and it does not expire, he cannot generate another Aapt “123456”, but a Capt “123456” is acceptable. He may also generate another Aapt “123456” after the previous one expires. It would not prevent another user (Uj, j≠i) generating an Aapt “123456” no matter whether Ui generates one or not.

The parameter k and the EQL also divides ACs into 2 states: dispensed and ready (see Table 3错误!未找到引用源。). Ready ACs can be used when their holder sends a query. Users still exchange ready ACs among their friends, but they do not add themselves in ACs’ EQ any more. Dispensed ones are not qualified to be used by current holder. Therefore, in order to make an AC useful, it should be exchanged for k times as fast as possible.

Table 4.9 AC states

| State | Explanation |
| --- | --- |
| Dispensed | EQL < k |
| Ready | EQL = k |

In order to manage ACs, each user maintains two lists: the dispense-list and the ready-list. It is obviously that dispense-list contains dispensed ACs and ready-list contains ready ones.

Table 4.10 AC storages

| List Name | Explanation |
| --- | --- |
| ready-list | It contains ready ACs. |
| dispense-list | It contains dispensed ACs. |

To avoid ACs generated by a creator simultaneously being dispensed to the same kth user, we employ the parameter Seg and dc. The dc is a counter which is initialized to a random integer in the interval [1, Seg] when a user received an AC. The AC should not be sent to the next friend until dc reaches 0, which enable users to split these ACs into about Seg batches.

Exchange sending process

Each AC is unique and hold by only one user at one time. When a user exchanges an AC to a friend, he loses the control of that AC and cannot give it to another one. We discuss the process that users exchange ACs with their friends. The process that a user (Ui) exchanges ACs to his friend is shown in Figure 4.3 AC exchange process (send). When Ui encounters a friend (Uj), he would send 3 kinds of ACs to the friend, which include dispensed ACs, ready ACs and new ACs. Even though new ACs must be dispensed ACs, we discuss them separately. During the exchange process, Ui generates the specified number of ACs and selects a part of dispensed ACs and ready ACs, then he sends them to his friend. He also remembers whom he sends them to, which enables him to transmit replies to the friend.

Figure 4.19 AC exchange process (send)

Generate new ACs

Each user (Ui) has an obligation to generate proper number of ACs for their friends, so that he guarantees that the number of ACs created by him in the network is larger than DN after he exchanges ACs with a friend (Uj). We assume that he must give Uj Sg new ACs to achieve that goal. Ui initializes entries for his new ACs, i.e., Cid (his identity), Capt (a new appointment number), Clct (his current location), Aid (A default value Aiddefault), Aapt (The same value as the Cid), time-out. The EQ in a new AC is initialized to empty, and dc is 0. We assume that the number of Ui’s ACs that are remained in the network is equal to RNi. The method of determining RNi is discussed in 0. Then .

Pick dispensed ACs

Ui decreases dcs of all his dispensed ACs, so that he gets Sd ACs whose dc values are equal to zero. They should be sent to his friend. Since the parameter dc is initialed as an integer between 1 and Seg randomly, there are about  dispensed ACs eligible to be sent to the friend. That enables us to separate ACs which come from a common creator.

Pick ready ACs

There are two ways for a user to provide ready ACs: 1. give his own ready ACs to friends directly; 2. dispensed ACs whose EQL values are equal to k-1. Since ready ACs are precious for every user, the strategy that a user shares ready ACs with his friends should save his own ready ACs. the number of ready ACs which would be sent to the friend is

, where Sd,k-1 denotes the number of dispensed ACs whose EQL values are equal to k-1. We assume that Uj has no ACs at all when they encounter. After their exchange, if Uj has more ready ACs than Ui, Ui must have all his old ready ACs with him; if Ui loses some of his ready ACs, Uj does not have more ready ACs than Ui. In another word, Ui should do his best to help his friend Uj, but Ui is not in a dilemma worse than his friends. That strategy makes users who sends more queries than friends supplement their AC consumption quickly, while protecting their friends’ profits. Then Ui picks Sr ready ACs randomly and sets their dcs to 0.

Send ACs

An AC should be sent to Uj, if and only if its dc is equal to 0. Before it is sent to Uj, Ui adds its identity to the its EQ. Besides, Ui replaces its Aid and Aapt with his own identity and a new appointment number. The old Aid is recorded in his agent-list, so do both old and new Aapts. The structure of the agent-list is shown in Table 5. When the AC comes from his dispense-list, he checks its EQL. If it is equal to k and the last entry of EQ is himself, he generates a unique pseudonym for the AC (Psd) and records the pseudonym in the agent-list (Nxt). Otherwise, he records the identity of his friend in Nxt. Since items which is relevant to an expired ACs would be deleted, each item in the agent-list is related to a distinct existing AC. Information in it can be used for statistics showing in the following sections.

Table 4.11 Agent_list entries

| List Name | Explanation |
| --- | --- |
| Aidold | The previous agent identity. |
| Aaptold | The previous agent appointment number |
| Aaptnew | The current agent appointment number |
| Nxt | The identity of the next friend. |
| time-out | The time when the AC expires |

Get the number of ACs

Ui can get the number of existing ACs which is generated by him based on the information in his agent-list. He counts the items in the list whose Aidold value is equal to Aiddefault, that is the RNi.

Check duplicated appointment numbers

All existing appointment numbers generated by a user are recorded in his agent-list. If Aidold value is equal to Aiddefault, the Aaptold is a Capt; Otherwise, the Aaptnew is a Aapt. When the user generates a new appointment number, he must check the agent-list to guarantee it is unique.

Exchange receiving process

The process that Ui receives ACs from his friend is shown in Figure 4.4 AC exchange process (receive). ACs are divided into two groups based on their EQL values, which are dispensed ACs and ready ACs. The dc value of a dispensed AC is assigned to a random integer in the interval [1, Seg], then the AC is moved into his dispense-list. Ready ACs are moved into his ready-list, and their dc values are set to -1.

Figure 4.20 AC exchange process (receive)

Transmit replies

When a user receives a reply, he checks his agent_list. If the sender identity is the LBSP, he searches for a record in his agent_list, whose Aidold is Aiddefault and Aaptold is equal to the Aapt in the reply. If the sender identity is a user, he looks up for an item whose Aidold is equal to the sender identity and Aaptold is equal to the Aapt in the reply.

Based on the record he finds, he modifies information in the reply. The sender identity is set to his own identity; the destination is set to the user whose identity is Nxt; Aapt is assigned to Aaptnew. The item related to that AC can be deleted after the replacement, then he replies to the next friend (Aidnew).

After the original requester sends the query out, he claims himself as the one whose identity is Psd. That identity is equal to the Nxt in the kth user’s agent_list. As a result, nobody knows the identity of the original requester. The process is shown in Figure 4.5.

Figure 4.21 Transmit reply

Example

We assume there are four users in our social network: Alice, Bob, Charlie and David and there are 3 pairs of friends (i.e. Alice and Bob, Bob and Charlie, Charlie and David) in our example. We assign the system parameter k to 3, so that each Appointment Card must be exchanged for three times before it can be used. The system parameter DN is equal to 100, so that each user guarantees there are more than 100 ACs generated by him in the network.

When Alice encounters Bob, both of them generate 100 new ACs and give all the new ACs to the other one. Table 6 shows the number of ACs hold by Alice and Bob after the exchange where ACAlice,1 denotes the ACs generated by Alice and with EQL equal to 1.

Table 4.12 Alice and Bob exchange ACs

| Users | Alice | Bob |
| --- | --- | --- |
| Dispensed ACs | 100 ACBob, 1 | 100 ACAlice, 1 |

Then Bob encounters a friend Charlie who is a new user. Since Bob has already generated 100 ACs and none of them are used, he generates no AC for Charlie. But, Bob should give half of his dispensed ACs to Charlie while Charlie generates 100 new ACs and give them all to Bob.

Table 4.13 Bob and Charlie exchange ACs

| Users | Bob | Charlie |
| --- | --- | --- |
| Dispensed ACs | 100 ACCharlie, 1 50 ACAlice, 1 | 50 ACAlice, 2 |

When Charlie encounters David, Charlie gets 100 new ACs from David. David get 25 ACs which are generated by Alice from Charlie. The 25 ready-ACs hold by David can also be exchanged to other users, as shown in Table 8.

Table 4.14 Charlie and David exchange ACs

| Users | Charlie | David |
| --- | --- | --- |
| Dispensed ACs | 100 ACDavid, 1 25 ACAlice, 2 |  |
| Ready ACs |  | 25 ACAlice, 3 |

The route of one of the ready ACs (for example, the ACAlice, 3) after it is received by David is shown as follow. For convenience, we donate it by ACx. David gives a pseudonym (Say p) to ACx before he exchanges it to another user. We assume that David gives it to Elizabeth and Elizabeth gives it to Franklin, and Franklin would use the ACx to send the query. Franklin sends the query pretending he is Alice and receives the reply pretending he is p. More specifically, in the replying phase, LBSP replies to Alice and then the reply would be transmitted by Bob, Charlie and received by David. After that, David broadcast the reply to p, other users would ignore that reply while Franklin realized it is the reply of his query since he uses the AC with pseudonym p.

Table 4.15 The exchange history of ACx

| User name | Cid | Capt | Aid | Aapt | Pseudonym |
| --- | --- | --- | --- | --- | --- |
| Alice | Alice | 233 | Alice | 557 | -- |
| Bob | Alice | 233 | Bob | 743 | -- |
| Charlie | Alice | 233 | Charlie | 691 | -- |
| David | Alice | 233 | David | 283 | p |
| Elizabeth | Alice | 233 | David | 283 | p |
| Franklin | Alice | 233 | -- | -- | -- |

When Franklin wants to send a query to a LBSP, he uses Alice’s identity and attaches the Capt (i.e. 233) in the query. Then he uses the pseudonym p as his identity when he receives a reply.

The LBSP replies to Alice, and the Capt (i.e. 233) is included in the message. When Alice receives the message, she checks her agent_list and finds the entry whose Aidold is equal to Aiddefault and Aaptold is equal to 233. Based on the entry, she attaches the Aaptnew (i.e. 557) in the reply and sends it to Nxt (i.e. Bob). We collect entries which are related to the ACx in Table 10. Then Alice remove the item from her agent_list. Also, the number of ACs generated by her decreases to 99 (=100-1), so that she should generate a new AC when she encounters a friend.

When Bob receives the reply from Alice, he also checks his agent_list and learns that the reply is related to the ACx based on the sender identity (i.e. Alice) and the Aapt (i.e. 557). He modifies the Aapt to 743 in the reply message and relays it to Charlie. Charlie also follows the same process. Different from the previous 3 friends, David relays the message to a pseudonym p, because the Nxt value is equal to p in his agent_list. Since Franklin is using the pseudonym p as his identity, he can get the reply message at last.

Table 4.16 Users' agent_list

| User Identity | Aidold | Aaptold | Aaptnew | Nxt |
| --- | --- | --- | --- | --- |
| Alice | Aiddefault (LBSP) | 233 | 557 | Bob |
| Bob | Alice | 557 | 743 | Charlie |
| Charlie | Bob | 743 | 691 | David |
| David | Charlie | 691 | 283 | p |

Table 4.17 Important information in the reply messages

| Transmit | Sender | Receiver | Aapt |
| --- | --- | --- | --- |
| LBSP → Alice | LBSP | Alice | 233 |
| Alice → Bob | Alice | Bob | 557 |
| Bob → Charlie | Bob | Charlie | 743 |
| Charlie → David | Charlie | David | 691 |
| David → p | David | p | 283 |

In a nutshell, the Aapt in the reply is the key for each user (friend) to relay the reply message to the next one. After a user relays the reply message to the next one, he removes the entry from his agent_list.

Vertex reduce algorithm

Movement of network nodes is a significant factor for the performance of DTNs. To evaluate the performance of different DTN protocols, researchers presented different kinds of movement models, like in [2]. Since shortest path algorithm is an essential and time-cost part of creating many movement models, it is an important topic to optimize the time-cost for simulators calculating the all-pairs shortest path problem in the entire map.

The best known non-negative edge weight undirected map all-pairs shortest path algorithm [3] has a complexity On2logn, which is still expensive when n is huge. For most real-world city map, there are thousands of points in a single square kilometer. Since a tens square kilometers map is reasonable for a DTN protocol evaluation, a simulator must cut down the time-cost of the calculation of the all-pairs shortest path.

Real-world map developers often use short straight lines to present curves, like in [4], so that a several-meters curves may contain tens of points. It is obviously that we do not need to calculate shortest path for every one of these points. In this paper, we present an Vertex Reduce Algorithm (VRA) to reduce the number of points before the process of the shortest path algorithm. The basic idea is that VRA removes all points whose degrees are less than 3 from the map, while keeps the result of all-pairs shortest path algorithm correct.

The rest of this paper is organized as follows: 5.1 presents some basic lemmas and definitions. The process of VRA is described in 5.2 and 5.3.

Ignorable vertex and reserved vertex

Vertices in the graph are considered as ignorable and reserved ones. In each iteration of VRA, we remove the ignorable vertex from the graph, while the reserved vertices compose a new graph.

Definition: the vertex whose degree is larger than 2 is the reserved vertex (R).

Definition: the vertex whose degree is smaller than or equal to 2 is called the ignorable vertex (G).

Definition: if two reserved vertices (e.g. R1,R2) are connected by a sequence of ignorable vertices (e.g. G1,G2,⋯,Gn), then the sequence from R1 to R2 (i.e., R1,G1,G2,⋯,Gn,R2) is a line-segment (LS).

A reserved vertex can belong to different line-segments, while an ignorable vertex belongs to a unique line-segment. Both the reserved vertex and the ignorable vertex are called vertices (V)

Definition: the shortest route between two vertices inside a line segment is called the inner shortest path (SPI), Let SPIVi,Vj denote the inner shortest path between two vertices (i.e. Vi,Vj) inside a line segment.

Lemma1: If two vertices (i.e. Vi, Vj; i<j) are in the same line-segment, the shortest path between them is

SPVi,Vj=minSPIVi,Vj,SPIR1,Vi+SPIVj,R2+SPR1,R2

Since an all-pairs SPI has a complexity equal to or smaller than On and the n here is much fewer than the number of points on the entire map, the time-cost for SPI is ignorable comparing to the time-cost of the entire map all-pairs shortest path calculation.

Lemma2: We assume that there are two different line-segments (i.e., LSi=Ri1,Gi1,Gi2,⋯,Gin,Ri2 and LSj=Rj1,Gj1,Gj2,⋯,Gjm,Rj2). We pick a vertex Vi from LSi and a vertex Vj from LSj, then the shortest path between Vi and Vj is

SPVi,Vj=minSPVi,Ri1,Rj1,Vj,SPVi,Ri2,Rj1,Vj,SPVi,Ri1,Rj2,Vj,SPVi,Ri2,Rj2,Vj

where SPVi,Ria,Rjb,Vj=SPVi,Ria+SPRia,Rjb+SPRjb,Vj. Here Vi and Ria are in the same line-segment, so we can use lemma1 to calculate SPVi,Ria, so does SPRjb,Vj. The SPRia,Rjb is the only part we need to calculate using all-pair shortest path algorithms. We should notice that Ria and Rjb could be the same vertex, but it does not make any difference to the lemma.

Vertex reducing

VRA iteratively removes ignorable vertices from the graph until only reserved vertices remained. In each iteration, we remove all ignorable vertices from the graph but keep the edges. If there are more than one routes between a pair of reserved, we keep the shortest one and remove others.

Remove ignorable vertices

Figure 5.1 Remove ignorable vertices

Since the degree of ignorable vertices is no more than 2, an ignorable vertex has only 0, 1 or 2 neighbours. In the case of 0 or 1 neighbour, we simply delete the ignorable vertex; if it has two neighbours, we connect its two neighbours before it is removed, as shown in Figure 5.1. When we remove an ignorable vertex, the weight of the line which connects its two neighbours is equal to the sum of its recent two lines’ weights (i.e., Wl+Wr). After we remove all ignorable vertices in a line-segment, the two reserved vertices at the ends of the line-segment are connected with a line directly, whose weight is the sum of all the intermediate ones’ (i.e., W1+W2+W3+W4).

Tidy reserved vertices connections

After we remove all ignorable vertices, all the reserved vertices are connected directly. However, it is possible that there are several connections between a pair of reserved vertices. The shortest route between a pair of neighbour vertices makes other longer ones redundant obviously, so that we

Figure 5.2 Tidy reserved connections

remove all routes except the shortest one, as shown in Figure 5.2. We assume that W2 is the shortest line in their three connections, then W1 and W2 are removed.

Iterations

Algorithm 1 vertex reducing

function reduce(Graphi)

Copy Graphi to Graphi+1

for every ignorable vertex Gu in Graphi+1 do remove(Gu) (see 5.2.1)

for every pair of reserved vertices do remove redundant connections (see 5.2.2)

end function

function VRA(Graph0)

i=0

while i=0 or there are any differences between Graphi-1 and Graphi

do

call reduce(Graphi)

i=i+1

end

end function

The whole algorithm is shown in Algorithm 1. The input of the algorithm is the original entire map, which is called Graph0. In the ith iteration, VRA makes a copy of Graphi as Graphi+1. If i>0, the Graphi is the output of the previous (i.e., i-1th) iteration. VRA removes all ignorable vertices in the Graphi+1 as described in 5.2.1 and remove all unnecessary routes between reserved vertices as described in 5.2.2. In other words, Graphi is smaller and smaller as i increases, because we always remove ignorable vertices from them. If Graphi is equal to Graphi+1, which means that the ith iteration makes no modification on the graph, the algorithm ends.

Vertex assembling

Algorithm 52

function assembleGraphi

for all ignorable vertices in Graphi as Gu

for all other vertices in Graphi as Gv

if Gu and Gv are in the same line-segment then

Use lemma 1 to calculate their shortest path.

else

Use lemma 2 to calculate their shortest path.

endif

end

end

end function

function AssembleAll(i)

while i > 0 do

call assembleGraphi-1

i=i-1

end

end function

We assume that the vertex reducing process stops at the ith iteration. Then the Graphi is the input of an all-pairs shortest path algorithm. Let SPiRu,Rv denote the shortest path from Ru to Rv in Graphi. We can infer the all-pairs shortest path of Graphi-1 based on Graphi with lamme1 and lamme2. The algorithm ends when we get the result of Graph0. The algorithm is shown in Algorithm 52.

We start from the Graphi-1 and assemble all intermediate graphs, until we get the result of Graph0. For any intermediate Graphj, we have already get the all-pairs shortest path result of its reserved vertices in the previous iteration (i.e. Graphj+1). Then we calculate the shortest path from any ignorable vertices to all others. When the two vertices are in the same line-segment, we use lemma1 to calculate their shortest path. The lemma 1 includes 4 parts: SPIVi,Vj, SPIR1,Vi, SPIVj,R2 and SPR1,R2. Since we already get the result of SPR1,R2 in the previous iteration, we just need to calculate 3 SPI parts, which is easy and has a small scale. If the two vertices are in different line-segments, we use lemma2 to calculate the shortest path. We also already get all the SP parts from the previous iteration so that we just need to deal with their SPI parts.

Finding nodes in a range

We assume that there are n nodes in a planar map (size: h×w). Those nodes keep moving on the map slower than a speed threshold (S). If the distance between two nodes a and b is smaller than a radius threshold (r), they are connected. We want to calculate all-pairs connections for all nodes on the map periodically.

If we calculate these connections directly, the complexity is On22. We assume that r≪minh,w and nodes are distributed on the entire map symmetrically, then the expected number of nodes which has a connection with a node (v) is nh×w×πr2≪n. If we traverse all other nodes (n-1) for v, it is a waste of time.

We propose an Nodes in the Range Algorithm (NRA) to optimize the calculation of all-pairs connections. we draw grids whose size is l×l on the map. when we calculate connections of the node v, only nodes in surrounding grids will be traversed.

Static nodes

Figure 6.1 grid size for still nodes

If nodes on the map are still, the radius threshold r is the only factor of the selection of the grid size (l×l). The length l of the side of a grid should be equal to r, as shown in Figure 6.1. For any node v in the dark grid, all nodes that the distance between them is equal to or shorter than r should be in the grey or dark grid. Therefore, if we want to get all connected nodes to node v, we just simply need to traverse all nodes in the grey and dark grids instead of all nodes in the entire map. In this case, the expected number of nodes which we need to traverse is 9n×r2h×w when we check connections for a node v. If we calculate connections for all nodes, we simply need to get the distances of about 9n2×r22×h×w pairs of nodes, instead of n22 pairs.

Mobile nodes

The problem is a bit more complicated if nodes are moving. Assuming that we want to calculate all-pairs connections at t1, first, we draw grids at time t0, so that we know which grid is a node in at t0. Instead of t0, we want to get connections at t1. In this case, the length l might not equal to the radius threshold r.

Suppose that we have two nodes u and v, and their distance is d1 at t1. Since the maximum speed of nodes is S, their distance d0 at t0 satisfies

max0,d1-2δ≤d0≤d1+2δ

Figure 6.2 length of grid

where δ=St1-t0. When d1≤r, these two nodes are connected at t1, then d0 must satisfy 0≤d0≤r+2δ. If they are in the same or adjacent grids at t0, as shown in Figure 6.2. We have l=r+2δ, so that ∀d0∈0,r+2δ: d0≤l. In other words, there is no grid between two nodes at t0.

Complexities

The complexity of drawing grids is On, while that of checking connections is O9n2×l22×h×w, so the checking part is much more expensive than the drawing part. Since l=r+2δ=r+2St1-t0, the checking part achieve its best performance when t1-t0=0, then its complexity is O9n2×r22×h×w. That is equal to the complexity when nodes are still.

REFERENCES:

A. Lastname1, B. Lastname2, C. Lastname3 and D. Lastname4, “Article name,” in Publication name, vol. 13, issue 2, pp. 223–244, 2017.

K. Fall, "A Delay-Tolerant Network Architecture for Challenged Internets," in Proceedings of the Conference on Applications, Technologies, Architectures, and Protocols for Computer Communications, pp. 27-34, 2003.

J. Schiller, A. Voisard, "Location-Based Services," Morgan Kaufmann, 2004.

M. Gruteser, D. Grunwald, “Anonymous Usage of Location-Based Services Through Spatial and Temporal Cloaking,” in Proceedings of the 1st International Conference on Mobile Systems, Applications and Services, pp. 31-42, 2003

S. Zakhary, M. Radenkovic, "Utilizing social links for location privacy in opportunistic delay-tolerant networks," in Proceedings of International Conference on Communications (ICC), 2012.

S. Zakhary, M. Radenkovic, A. Benslimane, "The quest for location-privacy in opportunistic mobile social networks," in Proceedings of IWCMC, 2013.

S. Zakhary, M. Radenkovic, A. Benslimane, "Efficient Location Privacy-Aware Forwarding in Opportunistic Mobile Networks,"  IEEE Trans. on Vehicular Technology, Vol. 63, No. 2, pp. 893 – 906, 2014.

E. Daly, M. Haahr, “Social Network Analysis for Routing in Disconnected Delay-Tolerant MANETs,” in Proceedings of the 8th ACM International Symposium on Mobile Ad Hoc Networking and Computing, 2007.

M. Duckham, “Moving forward: Location privacy and location awareness,” in Proceedings of the 3rd ACM International Workshop on Security and Privacy in GIS and LBS, pp. 1-3, 2010.

T. Hashem, L. Kulik, “Safeguarding location privacy in wireless ad-hoc networks,” in Proceedings of  UbiComp, pp. 372-390,  2007.

L. Sweeney, "k-Anonymity: A Model for Protecting Privacy," International Journal on Uncertainty,. Fuzziness and Knowledge-based Systems, Vol. 10, No. 5, pp. 557-570, 2002.

M. Mano, Y. Ishikawa, “Anonymizing User Location and Profile Information for Privacy-aware Mobile Services,” in Proceedings of the 2nd ACM SIGSPATIAL International Workshop on Location Based Social Networks, pp 68-75, 2010.

T. Wang, L. Liu, “Privacy-aware mobile services over road networks,” in Proceedings of the VLDB Endowment, pp. 1042-1053, Vol. 2,  2009.

R. Lu, X. Lin, X. Shen, “SPRING: A Social-based Privacy-preserving Packet Forwarding Protocol for Vehicular Delay Tolerant Networks,” in Proceedings of IEEE INFOCOM, 2010.

J. Zhang, C. Chow, “REAL: A Reciprocal Protocol for Location Privacy in Wireless Sensor Networks,” IEEE Trans. on Dependable and Secure Computing, Vol. 12, No. 4, pp. 458 – 471, 2015.

P. Hui, J. Crowcroft, “How Small Labels create Big Improvements,” in Proceedings of PERCOM Workshop, pp. 65–70, 2007.

Diffie, W., Hellman, M., "New directions in cryptography," IEEE Trans. on Information Theory, Vol. 22, No. 6, pp. 644-654, 1976.

P. Jacquet, P. Muhlethaler, T. Clausen, “Optimized link state routing protocol for ad hoc networks,” in Multi Topic Conference, 2001.

R. Xiang, J. Neville, M. Rogati, “Modeling Relationship Strength in Online Social Networks,” in Proceedings of the 19th International Conference on World Wide Web, pp 981-990, April 26 - 30, 2010.

T. Spyropoulos, K. Psounis, C. S. Raghavendra, "Spray and Wait: An Efficient Routing Scheme for Intermittently Connected Mobile Networks," in Proceedings of the ACM SIGCOMM Workshop on Delay-tolerant Networking, pp. 252-259, 2005.

Working Day Movement Model

all pairs shortest path algorithm with expected running time O(n2logn)

https://www.openstreetmap.org/

Ari Keränen, Jörg Ott, Teemu Kärkkäinen, "The ONE simulator for DTN protocol evaluation," in Proceedings of SIMUTools 2009, Rome, Italy.